# Documentation

This document describes only the ideas and techniques used for this prototype. For details, you should refer to the source code (lib and src directories) and the scripts (BuildRobot.bat).

You may have to refer to other documentation on Lego Mindstorm (available on 28 April 2008):

http://www.penguin-soft.com/penguin/man/1/dll.html

http://www.penguin-soft.com/penguin/man/1/firmdl3.html

http://brickos.sourceforge.net/

http://mindstorms.lego.com/eng/products/ris/rissoft.asp

## Link the simulator with Bridgepoint

Bridgepoint offers a way to implement the external entities using java classes. Your project must be configured for that (refer to the Readme). When the project is configured, the Java classes that use the appropriate name (corresponding to the external entity key letters) and that implement the operations defined in the model are executed by the Model Verifier. Thus, each class that matches an external entity enables to execute your own code.

In our case, we need to implement PIO. So, we create a java class called PIO.java. Actually, this class calls native functions through SimuInterface.java. SimuInterface.java uses JNI (Java native Interface).

javah (provided in the Java SDK) generates the header file corresponding to that class:

#First compile the class

javac SimuInterface.java

#then, generate the header

cd ..

javah lib.SimuInterface

We get lib_SimuInterface.h. This header is implemented in the simulator (main.c). This implementation simply calls the simulator interface (lib_SimuInterface.c).

## Code generated for the simulator

The implementation of PIO_bridge.c is done so that the operations defined in the external entities call the corresponding operation in the simulator interface (lib_SimuInterface.c).

The only trick here is the implementation of TIM_bridge: we have to implement a different ETimer_msec_time function to match with Windows system calls. In our case we use gettimeofday and calculate manually the time passed from it.

## Code generated for the robot

Before you can send the loadable module to the robot you must patch the firmware:

cd sumo-on-target\bin

firmdl3.exe --tty=COM1 –f ..\brickos\boot\brickOS.srec

When your loadable module is compiled (it should be sumo-on-target\gen\sde\ rox.lx), you can load it on target.

bin\dll.exe -tCOM1 gen\sde\rox.lx

You can refer to Lego documentation for information on IR transfer. You RCX must be turned on and your tower (with battery) correctly connected to your computer. If you experience issues during the transfer (after 2 tries), try the -s option instead of the –f for firmdl3.exe. Should you still have problems, check the charge of your battery.

## Simulator interface

- The arrows keys (direction keys) can be used to move the robot on Z and X axis.

- The Page UP and Page Down keys can be used to rotate the robot.

- The ‘i’ key reinitialize the camera.

- The ‘m’ key enables or disables the motion of the robot.

- The ‘q’ key kills the simulator thread (it doesn’t kill the process).

- The ‘1’ key set to PRESSED or UNPRESSED the touch1 sensor (left).

- The ‘2’ key set to PRESSED or UNPRESSED the touch3 sensor (right).

- The left button of the mouse (plus mouse movement) rotates the scene on Y and X axis.

- The right button of the mouse (plus mouse movement) zooms on the scene.

- The middle button of the mouse (plus mouse movement moves the scene on X and Z axis.

## Known issues

You may have bad visualization and/or slow motion. In that case, configure your OpenGL settings. On the laptop provided by Ericsson under Win2K, I need to launch Windows Media Player to get normal motion (I know it is strange).

You may experience strange behaviors if a lot of tasks are running concurrently on your computer.

If you close the simulator using the cross in top-right of the window, the process is killed thereby if you run the simulator in Bridgepoint, Bridgepoint is killed.

In general, you may experience unexpected behaviors but the point of this simulator is to be a prototype. Take it as such: something that works enough to validate some concepts.